CS600/700 Series

Upgrading Guidelines  (V1.0.0)

# Run ‘DMR Upgarde Setup vx.xx.exe’ to install the upgrading tool.

# Install the USB driver if not

Then run the upgrading software tool, then

1)Open the upgrading object file;

2)Connect the radio to the PC by the programming cable;

3)Press the PTT & TopKey and then power up the radio;

4)Click ‘Download upgrade file’ button to start upgrading;

5)Wait and until the PC reminde Success or Failure (about 1minute);

6)Power Off the radio.

Note:

- Keep the cable connected stably and the radio is power on all the upgrading period.

- The upgarding object file starting with ‘D’ is for LCD version, whereas the ‘d’ for non-LCD version or the same name as your model.

3)  Press the PTT & TopKey and then power up the radio!